| ##### Ride-on Mower Safe Work Method Statement (SWMS) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Business Contact: |  |  | Phone #: |  |  |  | Principal Contractor (PC): |  |  |  |  |
| Responsible person (for monitoring SWMS and work): |  |  |  |  |  |  | PC Address: |  |  |  |  |
| Signature: |  |  |  | Date: |  |  | PC Phone #: |  | Date SWMS provided to PC: |  |  |
| Contact Phone #: |  |  |  |  |  |  | Job Site Address: |  |  |  |  |
| Site Management Plan | Is the work associated with a Construction Project? Yes No |  |  |  |  |  |  | If yes – This SWMS must align with requirements of the Site Management Plan in place for the Construction Project. |  |  |  |
| This work activity involves the following “High-Risk Construction Work” (HRCW - identified in the Job Task column) |  |  |  |  |  |  |  |  |  |  |  |
| Confined spaces |  | Mobile plant movement |  |  |  | Demolition of a load-bearing structure |  |  |  |  | Asbestos disturbance |
| Using explosives |  | Diving work |  |  |  | Artificial extremes of temperature |  |  |  |  | Tilt-up or pre-cast concrete |
| Pressurised gas distribution mains or piping chemical, fuel or refrigerant lines energised electrical installations or services |  |  |  |  |  |  |  |  |  |  |  |
| Structures or buildings involving structural alterations or repairs that require temporary support to prevent collapse |  |  |  |  |  |  |  |  |  |  |  |
| Involves a risk of a person falling from 2m or more, including work on telecommunications towers |  |  |  |  |  |  |  |  |  | Work in a ceiling space (W.A. Only) |  |
| Working at depths greater than 1.5 metres, including tunnels or mines |  |  |  |  |  | Work in an area that may have a contaminated or flammable atmosphere |  |  |  |  |  |
| Work carried out adjacent to a road, railway or shipping lane, traffic corridor |  |  |  |  |  | In or near water or other liquid that involves the risk of drowning |  |  |  |  |  |
| WORKING WITH A RISK OF A FALL OVER 2 METRES. Select the fall from height 'hierarchy of control level' considered when establishing controls: |  |  |  |  |  |  |  |  |  |  |  |
| L 1: Work on the ground or solid construction |  |  |  |  | L 4: Use a fall arrest system e.g., safety harness, catch platforms |  |  |  |  |  |  |
| L 2: Use a passive fall restraint system e.g., guard rails, scaffolding, EWP |  |  |  |  | L 5: Implement administrative controls e.g., signage, or instruction |  |  |  |  |  |  |
| L 3: Use a work positioning system e.g., travel restraint, rope access |  |  |  |  | Other? |  |  |  |  |  |  |
| More than one of these measures to reduce risk can be used. For example, engineering controls like edge protection can be implemented with administrative controls like training and use of this SWMS, while wearing PPE (non-slip shoes). Please describe why it is not reasonably practicable to use higher-order control measures. E.g. The job is less than 5 minutes on a ladder. |  |  |  |  |  |  |  |  |  |  |  |

| Planning/Preparation | - Liaise with Principal Contractor to identify on-site safety systems and procedures - Establish supervisory and communication arrangements - Principal contractor to confirm emergency response procedures are in place. |  |
| --- | --- | --- |
| Hold Points | - Hold points identified and signed off before continuing work. |  |
| Training/Licence | - All workers to have a General Construction Induction Card - Relevant workers have relevant certificates of competency, licenses, and training. - Trained First Aider on site - All workers trained in site-specific emergency and evacuation procedures, SWMS, safe work procedures, and safety data sheets. |  |
| Worker duties and responsibilities | - Fit condition for work, i.e.,, no signs of fatigue, alcohol or drugs - Attend all site inductions/briefings - Comply with all site requirements, e.g. PPE, Traffic Management Plans (TMP) - Only carry out work related to the contract - Inspect completed work and report possible safety, environmental and quality matters to the supervisor. |  |
| Monitor/Review | - All people involved in the task must have this SWMS communicated to them before work commences - SWMS to be reviewed and amended if necessary, in consultation with relevant people after any near miss or incident - If additional site hazards identified, review this SWMS and amend control measures to suit - People, including workers, contractors and sub-contractors, affected by the revisions to this SWMS, must be informed ASAP - Give the principal contractor a copy of the revised SWMS - The site supervisor to monitor works against the controls stated in this SWMS - SWMS must be kept on-site and made available for inspection or review - Keep a record of this SWMS until the job is complete or for two years if involved in a notifiable incident - Regardless of any other factor, the person in control of the workplace must review this SWMS at least annually. |  |
| Site-Specific Notes: |  | Act, Regulations, Codes of Practice References: - Work Health and safety Act 2011 - Work Health and Safety Regulations 2017 - First Aid in the Workplace Code of Practice 2020 - Hazodous Manual Task Code of Practice 2019 - How to manage work health and safety Risks Code Of practice 2019 - Managing Electrical Risks in the Workplace Code of Practice 2019 - Managing Noise and Preventing Hearinig Loss at Work Code of Practice 2019 - Managing Risk of Falls at workplace Code of Practice 2019 - Managing Risk of falls in housing construction code of Practice 2019 - Managing Work Environment and facilities Code of Practice 2019 - Managing risk of Hazodus chemicals in the workplace code of practice 2019 - Managing the risks of plant in the workplace code of practice 2019 - Work health and safety consulation, coordination and cooperation code of practice 2019 |

| Personal Protective Equipment (PPE) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Foot Protection | Hearing Protection | High Visibility | Head Protection | Eye Protection | Face Protection | Hand Protection | Protective Clothing | Breathing Protection | Sun Protection | Safety Harness | Seat Belt |

| Job Task | Hazards | Risk | Control Measures |  |  | Responsible Person |
| --- | --- | --- | --- | --- | --- | --- |
| - Check weather conditions & prepare | - Infection - Disease - Bites - Stings - Fatality | 4A | - First Aid requirements: - Provision of first aid equipment - adequate Bites/Stings First Aid kit is available - Each worker must have access to the equipment - Workers have access to facilities for the administration of first aid - Ensure vaccinations (such as Hepatitis B, tetanus etc.) offered for staff - people with known allergic responses should consult a doctor and ensure appropriate kit is always available (Including EpiPen - in date). |  |  | Supervisor to maintain awareness of weather conditions Operator to follow controls |
|  | Extreme weather | 3H | - Check weather conditions – do not work in extreme conditions - Re-schedule works to work in more moderate temperatures. |  |  |  |
|  | Hyperthermia Hypothermia | 3H | - Workers trained to recognise the symptoms of hyperthermia and hypothermia |  |  |  |
|  |  |  | - Hot conditions. Ensure: - Suitable protective clothing - Sun brim on hard hats - Safety glasses - UV rated - Use 30+ sunscreen on exposed skin - Adequate drinking water - Access to shade on breaks - Adequate breaks. | - Cold conditions. Ensure: - Schedule warm-up breaks - Hold breaks inside - Dress warmly in layers - Stay dry (wet clothing chills the body rapidly) - Workers must get out of the cold as soon as they can if starting to feel symptoms - Alcohol, cigarettes, caffeine and certain medications increase susceptibility to cold. |  |  |
| - Working alone | Emergency | 4A | - Ensure a second person knows your location and expected time of return - Use mobile phone, telephone or radio to periodically check-in - Communication equipment within easy reach at all times - Take extra care to work safely and methodically - Have a first aid kit readily available and stocked appropriately. |  |  | Operator to maintain call log ins |
| - Arrival on-site & assess onsite conditions | Personal injury, property damage &/or environmental incident | 3H | - The vehicle should be positioned in a safe location, clear of traffic/vehicles/pedestrians during equipment delivery and removal (deploy physical barriers, caution signs as necessary) - Do not park illegally - Identify and obey all safety-related signage (check site entry requirements) - Report to Site Supervisor - Ensure site-specific induction undertaken - Assess mobile phone reception - The worksite is exactly as detailed in Terms of Agreement or contract - Complete a JSA specifying the control measures for unanticipated hazards. |  |  | Operator to check the site and conduct JSA where necessary |
| - HRCW Mower delivery & removal | - Crush injuries - Entrapment - Fatality | 4A | - Ensure the transport vehicle and/or trailer is appropriate - weight and size - Tie downs: - Tie-down points identified by tie-down decals where required - Use designated tie-down points on mower if provided (see operations manual) - Ensure mower stabilised before travel - Verify all tie-down equipment (chains, shackles etc.) is rated for the task and fixed in such a way to prevent movement during transit - Ramps: - Suitable load capacity for the weight of mower, wide enough, footboards on the side - Lock ramps in place, use locking pin/clips if required - Assess ground conditions at loading/unloading site to determine forward or reverse loading - Ensure clear of obstructions (tree branches, structures etc.) - Ensure people are clear of loading/unloading site - Load/unload as per manufacturer’s instructions: - Apply handbrake and block transport vehicle wheels - The mower is loaded/unloaded to and from the vehicle while on level ground - Aligned for loading/unloading and positioned with the vehicle by a suitably qualified, competent and experienced operator - Ensure mower is adequately restrained to prevent movement during transit. |  |  | Operator to ensure control measures followed |
| - HRCW Work area set-up | - Struck by motor vehicle - Working roadside - Injury to public & ground workers | 4A | - Park working vehicle in the driveway or allocated parking area to avoid travelling across roads when delivery working equipment - If loading/unloading/setting-up roadside – comply with local laws and permits - Barricading controls are implemented and authorised as part of the safe work system to protect people from hazards - Exclusion Zones established to stop people from entering the area - Signage in place to caution people not to enter - Public footpath or roadway exclusion zone: - Approval must be sought from the relevant authority (people should be safely directed to an alternative footpath where required) - Lane closures and other operations requiring barricades and signs erected should meet local road traffic authority, local government authorities’ requirements - Use a spotter as required - designate a responsible person for controlling the immediate area where the operation is occurring - Entry into the area must be authorised by the responsible person - The barricade must be far enough to keep observers safe - Ensure traffic management plan applicable to the type of work. Examples are as follows: - Freeway 100km/h, worksite between 1.2m and 9m = High Risk. Traffic control devices required (such as vehicle-mounted attenuators, lane closure adjacent to road works, reduced speed signs, flashing lights - Local traffic road 60km/h, worksite 1.2m = High Risk. See above controls - Works within 1.2m of any road = High Risk - Works further than 9m = Low risk. Lesser controls may be required. - For short term/mobile works, ensure the following: - Advance warning signs (i.e., “worker symbolic signs”) sufficient distance before works - Transition area (such as lane tapers – merging into a single lane), place cones approximately two times the approaching speed limit (in metres). Example: Road speed is 80km/h, tapering should begin at least 160m. Cones placed in a conspicuous location, evenly spaced – not more than 4m apart and at suitable diagonal into the lane - Clearance area (where the speed limit is more than 60km/h). Approx. 20-30m – no parked vehicles, plant or workers - Work Area (as required) - Termination (not required for short-term or mobile works) - Ensure suitable speed restrictions as required. Examples for short term works: - Between 1.2m-3m of a local road, the speed limit of 60km/h and barrier in place – no speed restrictions required. Where no barrier in place – reduce speed to 40km/h - For a freeway where a barrier is in place (within 1.2m-3m), reduce speed to 80km/h. Without barrier = 40km/h - Between 9-12m, no barrier = 80km/h on freeway - 12m+ = no speed restrictions - Ensure all signs placed in conspicuous areas where motorists can easily read them - Ensure all relevant workers wear suitable Hi-visibility/reflective vest. |  |  | Supervisor and operator to ensure control measures followed |
|  | Ejected debris | 3H | - Ensure blade covering guards are in place and are not damaged - Check the area and remove any objects which may be caught in the mower and ejected. |  |  |  |
| - Environment | Environmental impact | 3H | - Noise & vibration - Only operate within your approved operating hours - Be aware of maximum noise levels at the site - The plant is maintained to minimise noise - No noise produced outside council approved hours of operation. - Pollutants - Reduce pollutants by ensuring the transport vehicle and mower is tuned regularly and practice smooth driving techniques. - Fuels, oils & chemicals - Minimum amounts of hazardous substances are kept on site - Labelled and securely stored - Refuelling of vehicles/equipment will be undertaken at least 6m from drains and waterways - Follow SDS directions for disposal of chemicals in approved waste containers - A spill kit is available at all times & spills will be cleaned up immediately follow SDS directions - Waterways & soils - Do not wash out plant, equipment or containers where residue can enter waterways or drains - Vehicle movement - Follow the TMP, only travel on established tracks and roads - Use designated entry and exit points. - Native vegetation & weeds - Ensure that plant is washed free of dirt, vegetation, debris before travelling between sites. - Terrestrial fauna - No domestic animals on-site - Ensure all food scraps placed in lidded bins. |  |  | Supervisor and operator to ensure control measures followed |
| - House keeping | Slips, trips & falls | 3H | - Maintain housekeeping throughout the shift & clean-up - Clean up spills immediately - Ensure sufficient lighting to detect changes in level (using temporary lighting as required) - Do not jump from elevated edges >180mm, step carefully and use prepared access areas - Locate equipment where it will not become a tripping hazard - Ensure there is room to manoeuvre the equipment and no obstacles in the way - Check the end destination is prepared correctly for placement of the equipment - Ensure footwear is suitable. Snug-fitting shoes/boots with flat, non-slip soles, no loose soles, long laces, soles that are oily, or caked with mud etc. |  |  | Supervisor and operator to ensure control measures followed |
| - Manual tasks | - Fatigue - Prolonged seating | 3H | - Schedule adequate time between shifts for rest and recuperation - Schedule adequate breaks throughout the shift to allow for movement, e.g. go for a walk/one minute warm up/stretch and flex - Provide adequate lumbar support. |  |  | All workers to follow control measures |
| - HRCW Establish a communication system with ground workers | Injury to ground workers | 4A | - Follow manufacturer’s instructions, specific SWMS, spotter instructions, site rules and TMP - Amber beacon-light must be flashing while moving - Ensure ground workers and operators know and use on-site UHF channel to communicate - Worker consultation: - Establish an effective system of communication between the operator and ground workers - Relevant workers trained in the procedures involved before the work commencing - Groundworkers are instructed not to approach until the operator has agreed - Groundworkers are familiar with the blind spots of the vehicle and are instructed on set distances to maintain while in operation - Operators and ground workers are required to wear high-visibility clothing - Ensure no people or obstructions in the vicinity before start-up - Maintain the exclusion zone and cease work if approached. |  |  | Supervisors to check all controls in place and followed Operator to follow controls and make supervisor aware of any issues |
| - HRCW Mowing | Mower failure | 3H | - Pre-inspect the mower, as per manufacturer’s instructions, before operation - Do not use if any fault/damage/missing parts - Report immediately and follow tag-out/lock-out procedures. |  |  | Supervisors to check all controls in place and followed Operator to follow controls and make supervisor aware of any issues |
|  | Falls | 3H | - Face the mower whenever you mount and dismount the truck - Use the provided steps/handholds when entering or exiting the cabin - Maintain a three-point contact with the steps and handholds (2 feet & 1 hand/1 foot & 2 hands) - Never mount or dismount a moving vehicle - Do not jump off the mower. |  |  |  |
|  | Loss of control | 4A | - Do not mow wet grass – slippery surfaces lead to loss of traction - Ensure speed is adjusted to suit conditions and terrain - Avoid sudden braking wheels can lock up. |  |  |  |
|  | - Crush injuries - Entrapment - Fatality | 4A | - Adjust the seat to achieve full pedal contact when operators’ back is against the back of the seat - Wear seatbelt - ROPS in place as per relevant AS/NZS and manufacturer’s specifications - Ensure all body parts are always kept within the cabin when operating the vehicle - Start on a level grassed area only. Start-up as per manufacturer’s instructions and never leave operator seat with the engine running - Do not use any controls as handholds when you enter/exit the operator compartment - Ensure the vehicle travels at a safe distance from the edge of cliffs, pits, ponds, drop-offs etc. - When working on slopes: - Follow manufacturer’s recommendations for weight limits of towed equipment – especially when towing on slopes - Travel up and down, not across - Ensure slow and gradual movements - Avoid starting/stopping on slopes. If tyres lose traction, disengage blades and proceed slowly straight down the slope - There is no such thing as a safe slope. Be alert - Do not park on slopes. If necessary, ensure to chock, or block wheels. Use parking brake - Be familiar with the width of equipment and turning circle to maintain safe clearances |  |  |  |
|  | Injury to public | 4A | - Maintain awareness of people, tractor entering the operational zone and cease work immediately if the incursion is uncontrolled. Follow: - Manufacturer’s instructions; - Site safety rules; - TMP. - Ensure no people or obstructions in the vicinity before start-up - When reversing, take care that the area behind is clear of obstructions and bystanders - Use caution when approaching blind corners, shrubs, trees. that may block vision - Slow down before turning corners - Ensure no people in the vicinity before and during operation. Maintain exclusion zone and cease work if approached - Travel at an appropriate speed for conditions. Reduce speeds over unexpected hazards – such as wet conditions, long grass, hill peaks or low visibility. |  |
|  | - Cuts - Lacerations - Amputation | 3H | - Always keep body parts clear of moving blades - Whenever handling the blades always use protective gloves - Checking the blades: - Stop the engine and apply brakes - Before leaving the operator’s seat: - Disengage PTO Stop unit/shut off the engine - Remove key - Engage parking brake - Allow moving parts to stop - Release blockages with care. Keep all parts of the body away from the cutting edge - Beware of energy in the drive which can cause rotation when the blockage is released - Keep other people away from the blades. |  |  |  |
|  | Fire | 4A | - Ensure regularly maintained and properly inspected fire extinguisher carried at all times - Mowers are NOT driven through or near flammable vegetation - Monitor and observe alerts for not operating machinery on high fire risk days in the area - Consider re-scheduling operations - Regularly check: - Grass build up on the mower - Hot points on attachments, bearings and moving parts - To ensure a fire has not unintentionally started – use a spotter - DO NOT leave the mower unattended in long grass. |  |  |  |
|  | Ejected debris | 4A | - Adjust cutting height as required - Stop the blades rotating before crossing surfaces other than grass. |  |  |  |
|  | Noise | 3H | - Ensure workers not exposed to noise that exceeds the exposure standard (85dB averaged) - Hearing protection for operators is required, ensure: - Workers wear ear protection (earplugs are not sufficient protection by themselves) |  |  |  |
| - HRCW Refuelling | - Exposure to fuel - Inhalation - Ingestion - Eye & skin damage | 4A | - Avoid breathing vapours or contact with fuel - Fuel splashed clothing should be changed immediately - Do not siphon or swallow fuel as this can be fatal - Carefully wash hands and do not eat, drink or smoke during/after-handling fuel - Shower and wash immediately after work - Wash clothes in separate wash from other clothes. |  |  | Supervisors & Operator to follow controls |
| - HRCW On completion | Crush injuries/ entrapment | 4A | - Turn off all hydraulic valves and idle engine down. Apply the hand brake and remove keys - Inspect equipment for any damage/leaks. If any damage is detected – report immediately and take out of service. |  |  | Supervisor to confirm all workers have signed out Workers to comply with controls |
|  | Security breach | 3H | - If acceptable, remove or add barricades - All personnel sign-out on Site Register. |  |  |  |
|  | Vehicle/people impact | 4A | - Stay to designated access and egress routes - Maintain awareness of surroundings. |  |  |  |
| - Emergency response | - Injury - Fatality - Environmental damage | 4A | - For police, fire or ambulance call ‘000.’ - Follow site emergency and evacuation procedures - A communication system is available, e.g. a mobile phone or radio - Check for dangers to self before helping others - Maintain control of the area and stabilise the situation - Apply first aid to the injured worker - Complete an incident report. |  | - Refer to your SWMS implementing instructions for further specific emergency responses. | Supervisors and workers ensure controls followed |

| Overall Risk Rating after Controls | 1 - Low | 2 - Moderate | 3 - High | 4 - Acute |
| --- | --- | --- | --- | --- |

| Permits | Not applicable |  | Hot Work | Confined Space |  | Local council | Excavation |  | Lift (crane) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Working at height |  | Restricted access | Asbestos remediation |  | Other? | Other? |  | Other? |
| Plant & Equipment |  |  |  |  | Hazardous Substances |  |  | Supervisory Arrangements |  |
| Electrical tools and leads |  | Tested and tagged quarterly |  |  |  |  |  | Audits Spot Checks Reporting systems Suitably qualified supervisors for job Direct on-site supervision Remote site: communication systems/schedule Other? |  |
| Hand tools - Meet AS/NZS |  | Regular visual inspection |  |  |  |  |  |  |  |
| Ladders - Meet AS/NZS |  | Inspected and tagged out if damaged |  |  |  |  |  |  |  |
| Mobile plant-Ride On Mower |  | Inspected and tagged out if damaged |  |  |  |  |  |  |  |

| SWMS Sign-off |  | This SWMS developed in consultation and cooperation with workers and relevant organisation representatives. I have read the above SWMS, and I understand its contents. I confirm that I have the skills and training, including relevant certification, to conduct the task as described. I agree to comply with safety requirements within this SWMS, including risk control measures, safe work instructions, and PPE described. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Workers’ Name | Job Role / Position E.g. Supervisor, Worker, Trainee |  | Licences, competencies & qualifications (add as applicable) |  |  | Date | Signature |
|  |  |  | Type / Description | Class | Number |  |  |
|  |  |  | Construction Card |  |  |  |  |
|  |  |  | Construction Card |  |  |  |  |
|  |  |  | Construction Card |  |  |  |  |
|  |  |  | Construction Card |  |  |  |  |
|  |  |  | Construction Card |  |  |  |  |
|  |  |  | Construction Card |  |  |  |  |
|  |  |  | Construction Card |  |  |  |  |
|  |  |  | Construction Card |  |  |  |  |
|  |  |  | Construction Card |  |  |  |  |
|  |  |  | Construction Card |  |  |  |  |
|  |  |  | Construction Card |  |  |  |  |
|  |  |  | Construction Card |  |  |  |  |
|  |  |  | Construction Card |  |  |  |  |
|  |  |  | Construction Card |  |  |  |  |
|  |  |  | Construction Card |  |  |  |  |
|  |  |  | Construction Card |  |  |  |  |
|  |  |  | Construction Card |  |  |  |  |

| HIERARCHY OF CONTROLS |  |  |  | Most Effective |  |  |  |  |  |  |  | Least Effective |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Risk Matrix |  |  |  |  |  |  |  |  |  |  |  |  |
| Step 1: Determine Likelihood: What is the possibility that the effect will occur? |  |  |  |  |  |  |  |  |  | Step 2: Determine Consequence: What will be the expected effect? |  |  |
|  | Criteria |  |  |  |  | Description |  |  |  | Level of Effect | Example of each level |  |
| Almost certain | Expected in most circumstances. |  |  |  |  | The effect is a common result. |  |  |  | Insignificant | No effect – or so minor that effect is acceptable. |  |
| Likely | Will probably occur in most circumstances. |  |  |  |  | The effect is known to have occurred previously. |  |  |  | Minor | Minor first aid treatment,minimal business disruption, minimal environmental effect. |  |
| Possible | Could possibly occur at some time. |  |  |  |  | The effect could occur. I’ve heard of it happening before. |  |  |  | Moderate | Serious injuries, medium business interruption, medium environmental impact. |  |
| Unlikely | Unlikely but possible. |  |  |  |  | The effect could occur, but I have not heard of it happening before. |  |  |  | Major | Extensive injuries/fatality, significant business interruption, environmental harm, prosecution. |  |
| Rare | Highly unlikely to occur. |  |  |  |  | The effect is practically impossible. |  |  |  | Catastrophic | Multiple permanent disability/fatalities, business failure, substantial environmental harm, prosecution/imprisonment. |  |
| Step 3: Determine the risk score: |  |  |  |  |  |  |  |  |  | Step 4: Record risk |  |  |
| Effect Likelihood |  | Insignificant | Minor |  | Moderate |  | Major | Catastrophic |  | Score | Action |  |
| Almost certain |  | 2 Mod. | 2 Mod. |  | 3 High |  | 4 Acute | 4 Acute |  | 4A: Acute 3H: High 2M: Mod. 1L: Low | DO NOT PROCEED until risk level is reduced; immediately introduce high-level controls. Re-assess before proceeding. Review before commencing work. Introduce new controls and/or maintain high-level controls to lower the risk level. Controls are to be monitored frequently. Maintain control measures. Be caustious when working. Monitor and review regularly. Record and monitor. Proceed with work. Regularly review risks. |  |
| Likely |  | 2 Mod. | 2 Mod. |  | 3 High |  | 4 Acute | 4 Acute |  |  |  |  |
| Possible |  | 1 Low | 2 Mod. |  | 2 Mod. |  | 3 High | 4 Acute |  |  |  |  |
| Unlikely |  | 1 Low | 1 Low |  | 2 Mod. |  | 3 High | 4 Acute |  |  |  |  |
| Rare |  | 1 Low | 1 Low |  | 2 Mod. |  | 2 Mod. | 3 High |  |  |  |  |